Smartwire-DT library

The Library includes function blocks for PKE, NZM, DX-NET and DS7 Smartwire-DT modules. Following profiles are available:

PKE:

- Profile 1

- Profile 2

- Profile 3

NZM:

- Profile 1

- Profile 3

- (Profiles 2 und 4  will be included in a later version)

DS7

- Profiles 1 to 11

DX-NET

- Profiles 1 to 2

For every function block type, there are two versions. One version with a full output range and one with a reduced range (minimum). For the Smartwire-DT information, the full version has STRING (text), BOOL (Bit) and USINT variables. The reduced versions are without STRING and USINT outputs. The output range of that function blocks are much smaller.

For every function block a visualization is available, which can be connected with the function block in an easy way.

The input bytes of  Smartwire-DT profiles (IB0, IB1, etc.) have to be connected with the corresponding function block instance. The function block output range monitors the information, which is included in Smartwire-DT input bytes.  The connection from Smartwire-DT output (QB0) with the function block effects, that the commandos from the input range will reach the Smartwire-DT module, e. g. “contactor on” or “Activation of ZMR function”.

In a CoDeSys visualization you can choose a visualization element, which you can combine with the corresponding visualization page from the library, e. g. with “SWD_PKE_Profile2_gb”. After that you just have to connect the place holder with the function block instance (you can use F2 for that), which should be visualized, e. g. the instance “„PKE_Profile2_gb“ in the program „PKE_PRG_gb“. Then operation and view of the function block can be done with that visu page.

In the example projects, the Smartwire-DT input bytes are simulated and continuously change. Because of that the function block outputs are changing also all the time and the properties of function block and visualization can be watched.